This is the BSD driver documentation. The only functionality I left out was

some code and ioctl()s to write directly into video memory. If someone finds

they want this, it should be fairly easy to find the code in the original

FreeBSD sources (it is all #ifdeffed on something like METEOR_TEST_VIDEO),

hack it in, and get it working. I was not sure how it might work on Linux,

and it seemed like a possible security hole, and not relevant to the current

project, so I left it alone.

--Jim B.

Matrox Meteor Capture Card Driver

Meteor Driver Installation

--------------------------

To use the Matrox Meteor card in your system, you need a computer

that support the PCI (preferably the Type 2 or better) interface bus.

It is recommended that the system has as more than 16 MB of RAM since this

capture card directly deposits the image to system RAM.

The files required for Matrox Meteor card are:

/sys/pci/meteor.c

/sys/i386/include/ioctl_meteor.h (also know as:)

/usr/include/machine/ioctl_meteor.h

also the patch files:

meteor/usr/sys/i386/i386/conf.patch

meteor/usr/sys/conf/files.patch

meteor/sys/i386/conf/LINT.patch

These files are available for anonymous ftp at:

ftp://joy.cs.ndsu.nodak.edu/pub/meteor.tgz

1)  In the configuration file, add the line (as shown in

meteor/usr/sys/i386/conf/LINT.patch):

device meteor0

2) There is also a couple of optional parameters you may use

options "METEOR_ALLOC_PAGES=xxx"

specifies the number of contiguous pages to allocate when successfully

probed. The default number of pages allocated by the kernel is 151.

This means that there are (151*4096) bytes available for use.

options METEOR_DEALLOC_PAGES

deallocate all pages when closing the device. Note, the chance of

contiguously re-allocating new pages are very small.  The default

behavior is to not deallocate pages.

options "METEOR_DEALLOC_ABOVE=xxx"

deallocate all pages above the specified number. The default action is

to not deallocate above any pages.

3) Make and install the kernel.

4) Make the special file name:

mknod /dev/meteor0 c <major> 0

The major number is determined by the placement of the device in

conf.c. The patch supplied with the driver will make the major number

67.

Meteor Capture Modes

--------------------

The Meteor capture driver has three modes of capture operation.

1) Conventional read(2) interface.

This mode is the easiest and slowest to use. This mode is great for

capturing a single field at little programming cost.

In this mode, the user opens the device, set the capture mode

and size (See: METEORSETGEO ioctl call), and uses the read system

call to load the data into a buffer.

meteor_read.c read 400x300 RGB24 into a viewable PPM file

#include <sys/fcntl.h>

#include <machine/ioctl_meteor.h>

extern int errno;

#define ROWS 300

#define COLS 400

#define SIZE (ROWS * COLS * 4)

main()

{

struct meteor_geomet geo;

char buf[SIZE],b[4],header[16],*p;

int i,o,c;

if ((i = open("/dev/meteor0", O_RDONLY)) < 0) {

printf("open failed: %d\n", errno);

exit(1);

}

/* set up the capture type and size */

geo.rows = ROWS;

geo.columns = COLS;

geo.frames = 1;

geo.oformat = METEOR_GEO_RGB24 ;

if (ioctl(i, METEORSETGEO, &geo) < 0) {

printf("ioctl failed: %d\n", errno);

exit(1);

}

c = METEOR_FMT_NTSC;

if (ioctl(i, METEORSFMT, &c) < 0) {

printf("ioctl failed: %d\n", errno);

exit(1);

}

c = METEOR_INPUT_DEV0;

if (ioctl(i, METEORSINPUT, &c) < 0) {

printf("ioctl failed: %d\n", errno);

exit(1);

}

if ((c=read(i, &buf[0], SIZE)) < SIZE) {

printf("read failed %d %d %d\n", c, i, errno);

close(i);

exit(1);

}

close(i);

if ((o = open("rgb24.ppm", O_WRONLY | O_CREAT, 0644)) < 0) {

printf("ppm open failed: %d\n", errno);

exit(1);

}

/* make PPM header and save to file */

strcpy(&header[0], "P6 400 300 255 ");

header[2] = header[6]  = header[10] = header[14] = '\n';

write (o, &header[0], 15);

/* save the RGB data to PPM file */

for (p = &buf[0]; p < &buf[SIZE]; ) {

b[2] = *p++;		/* blue */

b[1] = *p++;		/* green */

b[0] = *p++;		/* red */

*p++;			/* NULL byte */

write(o,&b[0], 3);	/* not very efficient */

}

close(o);

exit(0);

}

2) Memory mapped single capture or unsynchronized continuous capture.

The single capture mode is designed for conferencing tools such as nv.

These tools need to control the starting of the image capture and also

need several frames a second. The continuous capture mode is designed

for applications that want free-running data.

In this mode, the user opens the device, set the capture mode

and size (See: METEORSETGEO ioctl call), memory maps the frame buffer

memory into the user process space, and issues either the

single-capture or the continuous capture call (See: METEORCAPTUR ioctl

call) to load the data into the memory mapped buffer.

As explained in the METEORCAPTUR ioctl call, the single frame capture

ioctl will block until the capture is complete, the continuous capture

will return immediately.

meteor_mmap_single_continuous.c

#include <sys/types.h>

#include <sys/mman.h>

#include <sys/fcntl.h>

#include <machine/ioctl_meteor.h>

extern int errno;

#define ROWS 480

#define COLS 640

#define SIZE (ROW * COLS * 2)

main()

{

struct meteor_geomet geo;

char buf[SIZE];

char *mmbuf;

int i,c;

if ((i = open("/dev/meteor0", O_RDONLY)) < 0) {

printf("open failed\n");

exit(1);

}

geo.rows = ROWS;

geo.columns = COLS;

geo.frames = 1;

geo.oformat = METEOR_GEO_RGB16 ;

if (ioctl(i, METEORSETGEO, &geo) < 0) {

printf("ioctl failed: %d\n", errno);

exit(1);

}

c = METEOR_FMT_NTSC;

if (ioctl(i, METEORSFMT, &c) < 0) {

printf("ioctl failed: %d\n", errno);

exit(1);

}

c = METEOR_INPUT_DEV0;

if (ioctl(i, METEORSINPUT, &c) < 0) {

printf("ioctl failed: %d\n", errno);

exit(1);

}

mmbuf=(char *)mmap((caddr_t)0, SIZE, PROT_READ, 0, i, (off_t)0);

#ifdef SINGLE_MODE

/* single frame capture */

c = METEOR_CAP_SINGLE ;

ioctl(i, METEORCAPTUR, &c);	/* wait for the frame */

/* directly access the frame buffer array data in mmbuf */

#else

/* continuous frame capture */

c = METEOR_CAP_CONTINOUS ;

ioctl(i, METEORCAPTUR, &c);	/* returns immediately */

/* directly access the frame buffer array data in mmbuf */

c = METEOR_CAP_STOP_CONT ;

ioctl(i, METEORCAPTUR, &c);	/* close will also stop capture */

#endif

close(i);

exit(0);

}

3) Memory mapped, multi-frame ring buffer synchronize capture.

This continuous capture mode is synchronized with the application that

processes up to 32 frames. This gives the advantages of both single and

continuous capture modes.

The kernel notifies the application of a new data by raising an

application defined signal. The driver also shares a structure with

the application that allows them to communicate which frame has been

written by the kernel and which frame has been read by the application.

The shared structure starts on the first page after your data. The

structure address can be found by calculation:

(number_rows * number_columns * pixel_depth + 4095) & 0xfffff000

or

((number_rows * number_columns * pixel_depth + 4095)/4096) * 4096

The shared structure is of type struct meteor_mem. The two most

important fields are called active and num_active_buf. active

is a bitmap of frames written by the kernel. num_active_bufs is

a count of frames marked in the active field. When a frame is read

in by the driver, the num_active_bufs count is tested, if this

count is below the threshold of number of active frames (value

in meteor_mem's hiwat variable), the bit representing frame

number in the buffer is stored in the active variable, the

num_active_bufs is incremented, the kernel then raises the specified

signal to activate the user application. The user application's

responsibility when getting the signal is to check the active bitmap

to determine the lowest active frame, use the data as the application

desires, clear the bitmap entry for that frame, and decrement the

num_active_bufs. If the threshold of number of active frames (hiwat)

has been exceeded, no new frames or signal from the kernel will occur

until the num_active_bufs is less than or equal to lowat.

The driver loads the frames in a round-robin fashion. it is expected

that the user removes them in the same order. The driver does not

check to see if the frame is already active.

The frame_size and number of frames in the buffer are also provided

to the meteor_mem structure, but changing these fields in the

application will not change the operation of the driver.

In programming for this mode, the user opens the device, sets the

geometry, mmaps the data/common control structure, then starts the

continuous capture mode. A special signal catcher is required to

process the frames as they are read by the kernel.

When specifying the geometry (See: ioctl METEORSETGEO), it

is important that the number of frames is set greater than 1.

#include <sys/types.h>

#include <sys/mman.h>

#include <sys/fcntl.h>

#include <sys/signal.h>

#include <machine/ioctl_meteor.h>

int video;	/* made global if you wish to stop capture in signal handler */

caddr_t data_frames;

struct meteor_mem *common_mem;

extern int errno;

#define FRAME_MAX

void

usr2_catcher()

{

#ifdef SIGNAL_STOP

struct meteor_capframe capframe;	/* for ioctl */

#endif

char *frame;

/* find frame */

frame = (char *) (data_frames + sig_cnt * common_mem->frame_size) ;

/* add frame processing here */

/* deactivate frame */

common_mem->active &= ~(1 << (sig_cnt % 16));

common_mem->num_active_bufs--;

/* process next frame on next interrupt */

sig_cnt = ((sig_cnt+1) % FRAME_MAX);

#ifdef SIGNAL_STOP

if (some_condition_requiring_stopping) {

capframe.command=METEOR_CAP_STOP_FRAMES;

if (ioctl(i, METEORCAPFRM, &capframe) < 0) {

printf("METEORCAPFRM failed %d\n", errno);

exit(1);

}

}

#endif

}

main()

{

struct meteor_geomet geo;

int height, width, depth, frames, size;

struct meteor_capframe capframe;

if ((i = open("/dev/meteor0", O_RDONLY)) < 0) {

printf("open failed\n");

exit(1);

}

printf("test %d %d\n", errno, i);

height = geo.rows = 120;

width= geo.columns = 320;

frames = geo.frames = FRAME_MAX;

depth = 2;			/* 2 bytes per pixel for RGB*/

geo.oformat = METEOR_GEO_RGB16;

if (ioctl(i, METEORSETGEO, &geo) < 0) {

printf("METEORSETGEO failed %d\n", errno);

exit(1);

}

c = METEOR_FMT_NTSC;

if (ioctl(i, METEORSFMT, &c) < 0) {

printf("ioctl failed: %d\n", errno);

exit(1);

}

c = METEOR_INPUT_DEV0;

if (ioctl(i, METEORSINPUT, &c) < 0) {

printf("ioctl failed: %d\n", errno);

exit(1);

}

size = ((width*height*depth*frames+4095)/4096)*4096;

/* add one page after data for meteor_mem */

data_frames = mmap((caddr_t)0, size + 4096, PROT_READ | PROT_WRITE,

0, i, (off_t)0);

if (data_frames == (caddr_t) -1) return (0);

/* common_mem is located at page following data */

common_mem = (struct meteor_mem *) (y + size);

signal(SIGUSR2, usr2_catcher);		/* catch new frame message */

capframe.command=METEOR_CAP_N_FRAMES;

capframe.signal=SIGUSR2;

capframe.lowat=12;		/* must be < hiwat */

capframe.hiwat=14;		/* must be < FRAME_MAX */

/* start the sync capture */

if (ioctl(i, METEORCAPFRM, &capframe) < 0) {

printf("METEORCAPFRM failed %d\n", errno);

exit(1);

}

/* this is the background working area, or you can sleep */

/* to stop capture */

capframe.command=METEOR_CAP_STOP_FRAMES;

if (ioctl(i, METEORCAPFRM, &capframe) < 0) {

printf("METEORCAPFRM failed %d\n", errno);

exit(1);

}

}

Meteor IOCTL Call and Parameters

--------------------------------

The Meteor capture driver has ioctl requests for capturing, reading card

status, for setting and reading the geometry, and for setting and reading the

attributes.

IT IS VERY IMPORTANT TO CHECK FOR ERRORS ON THESE RETURNING IOCTLs.

Errors indicate that something is very wrong with the ioctl and the

application should not attempt to proceed further with capturing. The

meteor capture driver still makes attempts to stop the next capture step if

an error occurred in a previous step but was ignored by the application

programmer.

1) ioctl requests METEORSETGEO and METEORGETGEO

METEORSETGEO and METEORGETGEO are used to set and read the input

size, input device, and output format for frame capture.

These ioctl routines use the meteor_geomet structure that has the

following entries:

rows		number of rows (lines high) in output image

columns		number of pixels in a row (width) in output image

frames		number of frames in buffer. Should be 1, unless using

the multi-framed synchronous capture mode (METEORCAPFRM)

which REQUIRES frames to be larger than 1.

Note: if rows, columns or frames is not changed, then

the existing values are used. The system defaults

is 640x480x1.

oformat		you may choose one of the following output format:

METEOR_GEO_RGB16	(RGB 16 bits xrrrrrgg gggbbbbb default)

METEOR_GEO_RGB24	(RBG 24 bits packed in 32 bits:

00000000 rrrrrrrr gggggggg bbbbbbbb)

METEOR_GEO_YUV_PACKED	(4-2-2 YUV 16 bits packed. byte format:

u0 y0 v0 y1 u1 y2 v1 y3 ...)

METEOR_GEO_YUV_PLANER	(4-2-2 YUV 16 bits planer format:

rows * columns bytes of y

rows * column / 4 bytes of even u

rows * column / 4 bytes of even v

rows * column / 4 bytes of odd  u

rows * column / 4 bytes of odd  v)

The METEORSETGEO ioctl will fail if more than one entry from a category

is selected. It is highly recommended that a METEORSETGEO is done

before capturing data because you cannot guarantee the initial mode

the card.

The METEORSETGEO will also attempt to reallocate a new contiguous

kernel buffer if the new geometry exceeds the old geometry. On

other hand, if the new geometry will fit in the existing buffer,

the existing buffer is used.

If METEORSETGEO fails the ioctl() will return a value of -1 and the

external variable errno will be set to:

EINVAL		invalid meteor_geomet structure pointer, rows, columns,

frames were invalid.

ENOMEM		could not allocate the contigous block.

2) ioctl requests METEORSFMT and METEORGFMT

METEORSFMT and METEORGFMT are used to set and read the camera input

standard format.

Possible formats are:

METEOR_FMT_NTSC		NTSC (default mode)

METEOR_FMT_PAL		PAL

METEOR_FMT_SECAM	SECAM

METEOR_FMT_AUTOMODE	Autodetect.

3) ioctl requests METEORSINPUT and METEORGINPUT

METEORSINPUT and METEORGINPUT are used to set and read the camera

input device. Using the DB9 connector on the Meteor card, 4 input

devices can be connected and an input camera can be selected with this

ioctl.

Possible formats are:

METEOR_INPUT_DEV0		(default if none specified)

METEOR_INPUT_DEV_RCA	(same as METEOR_INPUT_DEV0)

METEOR_INPUT_DEV1

METEOR_INPUT_DEV2

METEOR_INPUT_DEV_SVIDEO	(same as METEOR_INPUT_DEV2)

4) ioctl request METEORSTATUS

METEORSTATUS is used to read the status of the Meteor capture card

and returns the following information:

METEOR_STATUS_ID_MASK	4 bit ID of the SAA7196 scaler chip.

METEOR_STATUS_DIR	0 = scaler uses internal source.

1 = scaler uses external data of expansion bus.

METEOR_STATUS_OEF	0 = even field detected.

1 = odd field detected.

METEOR_STATUS_SVP	VRAM Port state:

0 = inputs HFL and INCADDR inactive.

1 = inputs HFL and INCADDR active.

METEOR_STATUS_STTC	0 = TV horizontal time constant (slow).

1 = VCR horizontal time constant (fast).

METEOR_STATUS_HCLK	0 = Horizontal Phase Lock Loop locked.

1 = Horizontal Phase Lock Loop unlocked.

METEOR_STATUS_FIDT	0 = 50 Hz Field detected.

1 = 60 Hz Field detected.

METEOR_STATUS_ALTD	0 = no line alternating color burst detected.

1 = line alternating color burst detected

(PAL/SECAM).

METEOR_STATUS_CODE	0 = no color information detected.

1 = color information detected.

5) ioctl request METEORCAPTUR

METEORCAPTUR is used to single frame capture or unsynchronized

continuous capture.

The single frame capture ioctl request will return only after a

frame has been captured and transfered to the frame buffer.

The unsynchronized continuous capture will return immediately and

data is directly deposited into the buffer when it is available.

Since this is unsynchronized, it is possible the data is being

written by the kernel while being read by the application.

These ioctl routines use the following settings:

following entries:

METEOR_CAP_SINGLE	capture one frame

METEOR_CAP_CONTINOUS	unsynchronized continuous capture

METEOR_CAP_STOP_CONT	stop the unsynchronized continuous

capture

If METEORCAPTUR fails the ioctl() will return a value of -1 and the

external variable errno will be set to:

EINVAL		invalid capture command value

ENXIO		there is not internal buffer to hold the frame.

this indicates the previous set geometry ioctl failed.

EIO		card is already capturing.

6) ioctl request METEORCAPFRM

METEORCAPFRM is used for synchronous capture of multiple frames.

This ioctl routines use the meteor_capture structure that has the

following entries:

command			possible values for command are:

METEOR_CAP_STOP_FRAMES	stop the capture does not use the

other variable in structure.

METEOR_CAP_N_FRAMES	start the capture using the other

variables in the structure as inputs

signal			signal to send to application when a new

frame has been captured. This signal will

only be raised if the captured frame is saved.

lowat			see below

hiwat			see below

When a new frame is completed, the driver checks the current unread

frame count stored in shared variable (the shared variable are stored

in the meteor_mem structure) num_active_buf, if the count is larger

than hiwat, the driver will not store any new frames and will not

send capture signal to the user application until the num_active_buf

is lower than lowat.

If METEORCAPFRM fails the ioctl() will return a value of -1 and the

external variable errno will be set to:

EINVAL		invalid meteor_geomet structure pointer or bad command.

ENXIO		there is not internal buffer to hold the frame.

this indicates the previous set geometry ioctl failed.

EIO		card is already capturing.

7) ioctl requests METEORSCHCV and METEORGCHCV

METEORSCHCV and METEORGCHCV are used to set and get the chrominance

gain control and effects the UV output amplitude.

If METEORSCHCV or METEORGCHCV fails the ioctl() will return a value

of -1 and the external variable errno will be set to:

EINVAL		invalid unsigned char pointer.

8) ioctl requests METEORGHUE and METEORSHUE

METEORGHUE and METEORSHUE are used to get and set the hue. The

signed character has legal values are from +127 which represent

+178.6 degrees to -128 which represents -180 degrees.

If METEORGHUE or METEORSHUE fails the ioctl() will return a value of

-1 and the external variable errno will be set to:

EINVAL		invalid signed char pointer.

9) ioctl requests METEORSCOUNT and METEORGCOUNT

METEORGCOUNT is used to get the count of frame errors, DMA errors and

count of the number of frames captured that have occurred since

the device was opened. METEORSCOUNT can be used to reinitialize the

counters..

This ioctl routines use the meteor_counts structure that has the

following entries:

fifo_errors	number of FIFO errors since device was opened.

dma_errors	number of DMA errors since device was opened.

frame_count	number of frames captured since device was opened.

If METEORSCOUNT or METEORGCOUNT fails the ioctl() will return a value

of -1 and the external variable errno will be set to:

EINVAL		invalid meteor_counts structure pointer.

Known Bugs:

----------

1) IIC register is difficult to set. We got around that by adding a long

wait at each IIC register write.

2) We had difficulties getting the Meteor capture card to work on systems

that used NCR chipset SCSI cards. It is possible that the Meteor and

NCR SCSI could work together using the newer TRITON motherboards.

Authors:

-------

Jim Lowe 	(james@miller.cs.uwm.edu)

Mark Tinguely	(tinguely@plains.nodak.edu)