COMTROL Corporation

2675 Patton Road

Roseville, MN  55115

C. Elmquist

November 7, 1994

THIS DOCUMENT UNDER CONSTRUCTION

What is the RocketPort toolkit ?

This is a preliminary release of the COMTROL RocketPorta

developers toolkit.  Use at your own risk and expect that a

future, formal release of these tools may differ

considerably from what you received here. The hardware is

cast in stone (well, silicon), so the functionality will not

change and if you build something from these tools it will

always work with all flavors of RocketPort currently known

to exist.

Traditionally, "toolkit" packages have been released as

libraries and compiled by their authors for each

and every possible platform that they want their product to

address.  Comtrol has chosen this time, to release the

toolkit in source code form and leave it up to the developer

to build it for his platform.  We feel this is a better,

open-architechure approach and won't limit the developer to

only the software platforms that we have time and resources

to address.  We want you to buy our boards and use them in

as many places as you can.  We put our "secrets" into

silicon and aren't afraid that you'll discover them by

knowing how to talk to our board.  We intend to make it as

easy as we can for you to develop software that makes use of

our RocketPort product.

How is this toolkit put together ?

This toolset is basically a large set of "C" source macros

which are intended to be included into your own source to

eventually build the functionality you require to control

one or more RocketPort boards.  There is also a small set of

"C" functions which do not implement well as macros. These

are also intended to be included into your code but could

also be built as a stand-alone library if desired.

The RocketPort card is completely I/O mapped, using no

shared memory or DMA channels.  Macros were used to build

this interface so that the final code would hopefully become

inline chunks of IN and OUT instructions rather than calls

to a library each time a RocketPort function was performed.

The idea is to make the code run as fast as possible without

having to know intricate details of every possible compilers

inline'ing technique.

You should cut and paste from this toolset, building your

own source modules which contain only the macros and C

functions that you actually use.

How does the RocketPort I/O mapping scheme work ?

The RocketPort's I/O mapping is sometimes a little tricky to

understand. Here's an attempt at explaining it in writing

(as opposed to one of many whiteboard drawings).  The

RocketPort board contains two flavors of ASIC.  One, called

the MUDBAC is the primary bus interface. It does the first

level of address decode and provides selects to the other

ASIC called the AIOP. The DIP switches on the card select

the I/O address of the MUDBAC and the MUDBAC occupies four

I/O addresses from this switch selected base address.

When initialization code is run, the MUDBAC is setup to

decode addresses for the AIOPs on the card. Each AIOP

contains eight serial ports and each port occupies eight I/O

addresses.  Therefore, one AIOP  occupies 64 I/O addresses.

In order to reduce the total number of contiguous I/O

addresses that a RocketPort board will occupy in the normal

I/O range of 0x100 to 0x3FF, COMTROL fully decodes all

twelve I/O address lines and intends that AIOPs on any one

board and MUDBACs across different boards will be "stacked"

or "aliased" on top of each other within I/O space.

What this means is that the first AIOP on a board will be

configured to reside at the users choosen I/O base address

for the board.  The second AIOP will be configured to reside

at the choosen address +0x400.  The third AIOP at +0x800 and

the fourth at +0xC00.  The MUDBAC on the first board will be

set by the DIP switch to reside at the users choosen I/O

address +0x40 (64 bytes past the first AIOP).   The MUDBAC

for the second board will be at the choosen address +0x440,

the third MUBAC at +0x840,

and the fourth at +0xC40. Printed documentation refers to

the choosen I/O address rather than the actual address

decoded by the switch. This is an attempt to similify this

addressing scheme from the users prospective. Here is a

typical user example:

The user has a 32 port board that he wants to reside at

0x180.  He sets the switches for the first board according

to the hardware reference card supplied with the board.

This addresses the first MUDBAC at 0x1C4.  Initialization

code then communicates with the MUDBAC, setting the first

AIOP at 0x180, the second at 0x580, the third at 0x980, and

2

the fourth at 0xD80.  The end result of this mapping is that

the board appears to take only 68 bytes from the normal I/O

range of 0x100 to 0x3FF.

If this user were to install another board, the hardware

reference card would instruct him to set the switches in

relation to the first board he installed.  What this does is

address the second MUDBAC +0x400 above the first one (or

0x540 in the above case).  A third board would have its

MUDBAC at +0x800 (or 0x940 in this case) and a fourth would

be at +0xC00 (or 0xD40).

Each of the second, third and fourth boards will need 64

bytes of I/O space to address their first AIOP.  Here the

user will need to be asked where he wants the "base address"

of these additional boards to reside.  Each board will

occupy 64 bytes from the range 0x100 to 0x3FF for the first

AIOP and then alias additional AIOPs above this range using

the +0x400, +0x800, +0xC00 technique.

One more way to look at this is the following pseudo code:

let b0 := user choosen base IO of board 0

then,

aiop0=    b0 + [0x000-0x03F]  set by software

aiop1=    b0 + [0x400-0x43F]       ""

aiop2=    b0 + [0x800-0x83F]       ""

aiop3=    b0 + [0xC00-0xC3F]       ""

mudbac=   b0 + [0x040-0x043]  set by DIP switch

let b1 := user choosen base IO of board 1

aiop0=    b1 + [0x000-0x03F]  set by software

aiop1=    b1 + [0x400-0x43F]       ""

aiop2=    b1 + [0x800-0x83F]       ""

aiop3=    b1 + [0xC00-0xC3F]       ""

mudbac=   b0 + [0x440-0x443]  set by DIP switch

let b2 := user choosen base IO of board 2

aiop0=    b2 + [0x000-0x03F]  set by software

aiop1=    b2 + [0x400-0x43F]       ""

aiop2=    b2 + [0x800-0x83F]       ""

aiop3=    b2 + [0xC00-0xC3F]       ""

mudbac=   b0 + [0x840-0x843]  set by DIP switch

let b3 := user choosen base IO of board 3

aiop0=    b3 + [0x000-0x03F]  set by software

aiop1=    b3 + [0x400-0x43F]       ""

aiop2=    b3 + [0x800-0x83F]       ""

aiop3=    b3 + [0xC00-0xC3F]       ""

mudbac=   b0 + [0xC40-0xC43]  set by DIP switch

Notice that all MUDBACs are referenced to the base address

of the first board.

3

We have built some structures into the rckt.c file which

make keeping track of MUDBACs and AIOPs much easier.  We

need to keep track of how many boards are in use and how

many ports exist on each board.  This translates into how

many MUDBACs and how many AIOPs we will have to deal with.

The macros are designed around the structures we have built

and will reference specific elements within these

structures. We will usually be indexing into arrays of these

structures by supplying a board pointer or channel pointer

when a macro is invoked.

RocketPort Hardware Details

Each AIOP is a custom, 36 MHz RISC processor that manages

eight full-duplex UARTs. The AIOP has more then enough

compute power to enable all eight UARTs to sustain 230.4Kbps

async simultaneously.

The AIOPs each have a private stash of highspeed static RAM

that allows each port to have a 1024 byte receive FIFO and

512 byte transmit FIFO.  The AIOP fully manages outgoing

flow control and in many applications can manage incoming

flow control as well.  There are various registers that can

be written or read to configure, control and monitor the

AIOP as it does its job.  Since the whole system is I/O

mapped, one of the features we exploite is the ability to

load the transmit FIFO by using a REP OUTS instruction on an

x86 host processor. Likewise, unloading the receive FIFO can

use a REP INS instruction.  The other thing we do is support

16-bit wide (word) transfers when doing these REP string

instructions.  So, when you have 28 characters to send to a

port, you can do it with 14 word transfers across the bus.

This can be a big win in performance since each ISA bus

transaction can be very time consuming.

The MUDBAC chip holds together all of the AIOPs on a board

and provides the interface to the host bus.  MUDBAC decodes

addresses for its own control via DIP switches and then has

programmable registers to configure the decoding for the

AIOPs it is connected to.  MUDBAC has a programmable

interval timer that can be used to issue periodic interrupts

to the host in the event a particular operating system does

not offer an appropriate polling mechanism to service the

board in a timely manner.  MUDBAC can also

be configured to manage interrupts to the host that are

generated by changes within the AIOPs.  The

MUDBAC/AIOP combination can be configured to interrupt the

host on TX FIFO empty, RX FIFO has

data, modem line status changes, parity errors, framing

errors, etc.  Macros within the toolkit help setup and

configure the devices for these various modes.

4

A RocketPort Driver

Included with this toolkit is a sample driver/application

program written in BORLAND C v3.1 which runs under DOS.  The

program doesn't really do anything important but it does

illustrate the sequence of things that your program or

driver must do in order to initialize, configure and

function a RocketPort card.  At this point, you are being

refered to the source of this sample program (sample.c) and

you should study it, compile it, port it to your platform,

etc. and begin building up your own set of RocketPort

functions.

The sample program sets up the RocketPort board to generate

a periodic interrupt (137 Hz) and it is during this

interrupt that data is moved to and from the ports on the

board.  Many operating systems provide this sort of polling

mechanism from the kernel directly and there many not be a

need for an additional hardware ticker.  You should consider

this when designing your driver.

Most of the C functions in rckt.c are not re-entrant.  You

must complete them in one thread before they get called

again or the state of the RocketPort board may be

compromised.

This toolkit is Comtrol's first attempt at a source level

toolset.  Please send comments (good or bad !) regarding the

structure, organization and usefulness of this package.  We

want to make the most usable tools available to you so that

you can easily integrate our products into your system.

As of this writing, COMTROL is in the process of improving

its Internet connection.  We will soon have FTP and WWW

services available to assist developers in any ways we can.

We expect to have shareable and contributed source codes

available as well as complete drivers and additional

development tools for all of our products.  We should be

online by the end of January `95.

5